RAFGSA - WINCH DRIVER TRAINING

Name: __________________________

No member is to undertake safety-critical tasks unsupervised before being certified competent by an authorised instructor.

| Preparation and Maintenance | Instructor Signature | Instructor Name | Date | Notes |
| --- | --- | --- | --- | --- |
| RAFGSA Winch Manual |  |  |  |  |
| Winch Daily Inspection |  |  |  |  |
| Tractor Daily Inspection |  |  |  |  |
| Tractor Driving |  |  |  |  |
| Winch Set-up |  |  |  |  |
| Winch Start & Warm-up |  |  |  |  |
| Use of Radio |  |  |  |  |
| Use of Stop Light |  |  |  |  |
| Winch Put Away |  |  |  |  |
| Logbook Completion |  |  |  |  |
| Winch Refuel |  |  |  |  |
| Tractor Refuel |  |  |  |  |
| Launching | Instructor Signature | Instructor Name | Date | Notes |
| Instruction in Technique |  |  |  |  |
| Competent to Launch Under Supervision |  |  |  |  |
| Competent |  |  |  |  |
| Emergencies | Instructor Signature | Instructor Name | Date | Notes |
| STOP Signal Received |  |  |  |  |
| Glider Wing Drop |  |  |  |  |
| Cable Break |  |  |  |  |
| Glider Over-runs Cable |  |  |  |  |
| Glider Enters Cloud |  |  |  |  |
| Glider Unable to Release |  |  |  |  |
| Cable Repair |  |  |  |  |

The above-named member has completed all relevant training and is authorised to perform the duties of a Winch Driver.

Signed: _______________		Instructor Name: _______________		Date: __________

(Please also complete the member's Ground Training and Qualification Record)

WINCH DRIVER TRAINING – GUIDANCE NOTES

Full Driving Licence

- Manual car licence (no automatic transmission restriction).

- Prerequisite for driving tractor.

- Without this qualification, member may drive winch but not tractor.

RAFGSA Winch Manual

- Read & understand.

- Discuss key points.

Winch Daily Inspection

- See Winch Manual.

- Ensure paperwork completed.

Winch prime mover inspection

- How to open bonnet.

- Fuel: at least half full. Oil: sufficient on dipstick. Coolant: top up if required. Tyres: correctly inflated. Wheels: bolts secure. General condition of vehicle: checked, free of FOD.

- Location of fluids / compressor.

- Remove drip trays before moving.

- Cold start: turn key against spring for glow plug. Start button on right-hand side of console.

Winch prime mover Driving

- Appropriate speed for towing winch.

- Reversing.

- Hand throttle: easier for maintaining constant speed over undulating terrain.

- Stopping engine: Pull stop lever. Hold out until engine completely stopped. Then push lever in and switch off key.

Winch Set-up

- Positioning. Angle accurately towards launch point.

- See Winch Manual for further details.

- Bus light signal & radio check.

Winch Start & Warm-up

- See Winch Manual.

Use of Radio

- Controls: on / off / vol, channel (usually Ch 2), push-to-talk switch.

- Callsigns: “Winch”, “Launch Point”, “(North / South) Retrieve”.

- Basic radio procedure: how to form a message. If in doubt: use plain English.

- Switch off radio and place on charge in workshop at end of day.

Use of Stop Light

- Indicates winch fault; winch unable to launch.

- Directional adjustment.

- After illuminating stop light, inform launch point of situation by radio.

Winch Put Away

- See Winch Manual.

Logbook Completion

- Number of winch launches & sign for master switch ‘OFF’.

Winch Refuel

- Use of LPG refuelling equipment; correct connection of LPG hose to winch tanks.

- Care of winch LPG filler dust covers. Fragile plastic – finger tight only.

EMERGENCIES

STOP Signal Received

- Immediate actions:

  - Throttle – IDLE.

  - Drive – NEUTRAL.

  - Drum Brake – stop cable.

- Subsequent actions:

  - Drum selector – NEUTRAL.

  - Assess situation:

    - Further launch attempt?

    - Retrieve vehicle to tow cable back to launch point? Cable may start moving without warning – hands clear.

    - Wind cable back in? Speak to launch point before moving cable.

Glider Wing Drop

- First priority: provide a stable platform for pilot to assess and respond to situation.

- Once the ALL-OUT signal is received at the winch, the winch driver is to continue the launch unless he receives a stop signal / message from the launch point or he/ she can see that the launch has failed.

Cable Break

- Cable can whip a significant distance following a break under tension. Keep winch doors closed during launching and don’t allow bystanders to loiter in vicinity of winch.

- Weak link failure only:

  - Cable visibly clear of glider: continue to wind in.

  - Cable not visibly clear of glider: immediately stop cable as above.

  - Try to observe strop landing area to assist recovery.

- Full cable break:

  - Cable visibly clear of glider: continue to wind in broken end.

  - Cable not visibly clear of glider: immediately stop cable as above.

  - Observe parachute landing area; if parachute / cable seen to fall across trees, road, airfield signs or buildings, alert launch point & retrieve drivers by radio.

- Low launch fail irrespective of cause: stop cable immediately to prevent glider fouling cable.

Glider Over-runs Cable

- Launch point should signal STOP.

- Proceed as above.

Glider Enters Cloud

- Continue to provide a stable platform for pilot to deal with situation.

- When release felt, reduce the power to idle to ensure separation from glider until parachute visible, and then recover as normal

Glider Unable to Release

- Immediate actions:

  - Throttle – IDLE.

  - Drum Brake – stop cable

  - Guillotine – operate.

  - Drive – NEUTRAL.

  - Drum selector – NEUTRAL.

- Subsequent actions:

  - Engine – OFF.

  - Cab – stay inside with doors closed until emergency over.

- Glider should open air brakes & fly steep turns above winch to generate slack in cable.

Cable Repair

- Practical demo.

- Ensure no-one moves cable while you’re working on it.